Q1: Convert the given high level code to corresponding risc-v code:  [5]

if (a > b):

a = a * d

elif (a == b):

b -= e;

else:

arr1[25] = b / e

Note: a, b, d, e variables are mapped to f4, f5, f6, f7 registers and all the numbers stored are single precision numbers. arr1 is a word array. The base address of arr1 is in register x20.

Answer:

Q2: Suppose a cache has 16 blocks. Each block can store 48 bits of data.

- What is the size of index bits?  [1]

- What is the size of offset bits?  [1]

- Using the mentioned cache system, for a memory address 1011100010, Find the index,

tag and offset bits.  [3]

Answer:

a)

b)

c) Index bits =

Offset bits =

Tag bits =

Q3: Multiply 12 and 13 using optimized multiplication.  Show the complete table.  [5]

Answer: